| # Router Prompt |  | # Commands to create basic router configuration |
| --- | --- | --- |
|  | Router> | Enable |
|  | Router# | Config t |
| G | ## Router(config)# | Hostname [ name of router] |
| G | ## Router(config)# | Enable secret [privileged mode password to be encrypted, i.e. class] |
| G | ## Router(config)# | IP host [name of router] [ IP addresses of router interfaces] |
| G | ## Router(config)# | IP host [name of router] [ IP addresses of router interfaces] |
| G | ## Router(config)# | No IP domain-lookup |
|  | ## Router(config)# | Interface e0 |
| I | ## Router(config-if)# | IP address [address of this interface] [subnet mask of this interface] |
| I | ## Router(config-if)# | No shutdown |
| I | ## Router(config-if)# | Interface s0 |
| I | ## Router(config-if)# | IP address [address of this interface] [subnet mask of this interface] |
| I | ## Router(config-if)# | No shutdown |
| I | ## Router(config-if)# | Clock rate 56000 ( if this interface has a DCE cable plugged in) |
| I | ## Router(config-if)# | Interface s1 |
| I | ## Router(config-if)# | IP address [address of this interface] [subnet mask of this interface] |
| I | ## Router(config-if)# | No shutdown |
|  | Router(config-if)# | EXIT ( to move back to global config mode) |
|  | ## Router(config)# | Router RIP |
| R | Router(config-router)# | Network [IP address of a directly connected network…no subnets!] |
| R | Router(config-router)# | Network [IP address of a directly connected network…no subnets!] |
|  | Router(config-router)# | EXIT ( to move back to global config mode) |
|  | ### Router(config)# | ### Line console 0 |
| L | ## Router(config-line)# | ### login |
| L | ## Router(config-line)# | ### Password [password to be used for entering user mode at login, i.e. cisco] |
| L | ## Router(config-line)# | ### Line vty 0 4 |
| L | ## Router(config-line)# | ### login |
| L | ## Router(config-line)# | ### Password [password to be used for entering user mode at login, i.e. cisco] |
|  | ## Router(config-line)# | ### Exit |
|  | ### Router(config)# | ### Exit |
|  | ### Router# | ### Copy run start |
|  | ### Router# | ### Show run (to verify configuration in RAM memory) |